承認書

Specification  For  Approval

| COMPANY NAME |  |
| --- | --- |
| CUSTOMER P/N |  |
| PRODUCT NAME | AC/DC ADAPTER |
| PARTS P/N | CP-0510(美规) |
| DATE | 2008-5-13 |

| ### APPROVED | ### REMARK |
| --- | --- |

寶三電子有限公司

POWSUN CO.,LTD.

總公司:臺灣臺北縣新莊市(五股工業區)五權一路13號5樓之一(504室)

TEL:(02)2299-7680     FAX:(02)2298-9686

工廠地址:廣東東莞清溪鎮漁梁圍朱屋村寶三電子廠

TEL:(0769)87389421  87389425  87389417

FAX:(0769)87389413

http://www.leadman.com.tw

http://www.powsun.com

E-mail:powsun@powsun.com

POWSUN CO.,LTD.

# SPECIFICATION FOR APPROVAL

1. FEATURE:

 INPUT       :UNIVERSAL  100-240 Vac/47-63  Hz  INPUT,  W I THOUT  ANY  SLIDE  SW I TCH

 OUTPUT    :+5V / 0-1A

 DIMENSION :50(D)*38(W)*21(H) mm

 EFFICIENCY :75% TYPICAL

 SAFETY :  FCC/CB /CE/ LVD

 EMI          :EN55022B

 HIGH FREQUENCY  DESIGN,  LESS  POWER  CONSUMPTION

 OVER VOLTAGE  PROTECTION,  SHORT  CIRCU ITED  PROTECTION,  OVER  POWER  PROTECTION

 CAN BE USED AT TEL COMMUNICATION, COMPUTER, INDUSTRIAL CONTORLLER & OA SYSTEM

2. INPUT:

| 2. 1 VOLTAGE | UNIVERSAL 100~240Vac,SINGLE PHASE |
| --- | --- |
| 2. 2 FREQUENCY | 47HZ~63HZ |
| 2. 3 CURRENT | 0.5A Max |
| 2. 4 INRUSH CURRENT | 6A Max/100Vac;12A Max/240Vac (COLD STABT AT 25℃ FULL LOAD) |
| 2. 5 EFFICIENCY | 75% TYPICAL (AT NOMINAL LINE VOLTAGE FULL LOAD) |
| 2. 6 INPUT POWER DISSIPATION | UNDER 0.5W < ZERO LOAD 115V/60HZ INPUT |
|  | UNDER 1.0W < ZERO LOAD 230V/50HZ INPUT |

3. OUTPUT:

| 3. 1 DC OUTPUT | VOLTAGE | +5.00V |
| --- | --- | --- |
|  | CURRENT | 1.0A Max 0.85A typ |
|  | REGULATION | 4.75V Min~5.0V Typ.~5.25V Max |
|  | RIPPLE & NOISE | 50mV Max |
|  | TOTAL POWER | 5W Max |

NOTE: FOR RIPPLE & NOISE MEASUREMENT, USE A 20MHZ BAND WIDTH FREQUENCY OSCILLOSCOPF, AND ADD A 0.1uF MULTILAYER CAP.  AND A LOW ESR ELECTROLYTICCAP. (10uF)  AT OUTPUT CONNECTOR TERMINALS. (AT NOMINAL LINE VOLTAGE FULL LOAD)

4. PROTECTION:

| 4. 1 OVER VOLTAGE PROTECTION | V out*(5.7V-6.2V) |
| --- | --- |
| 4. 2 SHORT CIRCUIT PROTECTION | AUTOMATIC RECOVERY AFTER SHORT-CIRCUIT FAULT REMOVED |
| 4. 3 OVER POWER PROTECTION | THE ADAPTER PROVIDE PROTECTION AT 110%-140% OF RATED OUTPUT POWER |

NOTE: WHEN PROTECTION CIRCUIT IS WORKING THE POWER SUPPLY WILL SHUTDOWN, ONCE THE ABNORMAL CONDITIONS ARE REMOVED. THE POWER SUPPLY WILL RESTART AUTOMATICALLY.

| APPROVED BY: | CHECKED BY: | PREPARED BY: |
| --- | --- | --- |

POWSUN CO.,LTD.

## SPECIFICATION FOR APPROVAL

5. SAFETY AND EMI REQUIREMENT:

5.1 SAFETY REQUIREMENT

a. SAFETY               :UL/TUV/CE/CCC/T-MARK

b. DIELECTRIC STRENGTH :10mA Max. CUT OFF CURREN

| (1):PRIMARY TO SECONDARY | : 2000Vdc FOR 1 MINUTE |
| --- | --- |
| (2):PRIMARY TO FRAME GROUND | : 1500Vac FOR 1 MINUTE |

c. INSULAION RESISTANCE:

| (1):PRIMARY TO SECONDARY | : 10 M OHMS FOR 500Vdc |
| --- | --- |
| (2):PRIMARY TO FRAME GROUND | : 10 M OHMS FOR 500Vdc |

5.2 EMI  REQUIREMENT  MEET  CISPR 22/FCC/VCC1, CLASS B

5.3 LEAKAGE  CURRENT  LESS  THAN 3.5mA

6.OPERTION & ENVIRONMENT PERFORMANCE:

6.1 TEMPERATURE RANGE

| OPERATING | 0℃ ~ +40℃ |
| --- | --- |
| STORAGE | -40℃ ~+70℃ |

6.2 HUMIDITY RANGE (NON CONDENSING)

| OPERATING | 10% ~ 90% RH |
| --- | --- |
| STORAGE | 5% ~ 95% RH |

6.3 COOLING                 SHOULD OPERATE WITHOUT FAN

7.M.T.B.F.:                50000 HOURS Min ; 40℃ SEE CONDITION

8.MECHANICAL:

8.1 WEIGHT:                 95g  Typ.

POWSUN CO.,LTD.

## SPECIFICATION FOR APPROVAL

( I ). LINE REGULATION TEST

Test result:

| Test condition | Spec. | Reading1 | Reading2 | Reading3 |
| --- | --- | --- | --- | --- |
| 90Vac / 50% load | 4.75~5.25V | 5.10V |  |  |
| 115Vac / 50% load | 4.75~5.25V | 5.10V |  |  |
| 132Vac / 50% load | 4.75~5.25V | 5.10V |  |  |
| 180Vac / 50% load | 4.75~5.25V | 5.10V |  |  |
| 230Vac / 50% load | 4.75~5.25V | 5.10V |  |  |
| 264Vac / 50% load | 4.75~5.25V | 5.10V |  |  |

(II). EFFICENCY TEST

Test result:

| Test condition | Spec. | Reading1 | Reading2 | Reading3 |
| --- | --- | --- | --- | --- |
| 115Vac / 100% load | 75% Typical | 75% |  |  |
| 230Vac / 100% load | 75% Typical | 74% |  |  |

(III). LOAD REGULATION TEST

Test result:

| Test condition | Spec. | Reading1 | Reading2 | Reading3 |
| --- | --- | --- | --- | --- |
| 115Vac / 0% load | 4.75~5.25V | 5.21V |  |  |
| 115Vac / 50% load | 4.75~5.25V | 5.10V |  |  |
| 115Vac / 100% load | 4.75~5.25V | 5.0V |  |  |
| 230Vac / 0% load | 4.75~5.25V | 5.21V |  |  |
| 230Vac / 50% load | 4.75~5.25V | 5.10V |  |  |
| 230Vac / 100% load | 4.75~5.25V | 5.0V |  |  |

(IV).RIPPLE & NOISE TEST

Test result:

| Test condition | Spec. | Reading1 | Reading2 | Reading3 |
| --- | --- | --- | --- | --- |
| 115Vac / 100% load | 50mV Max | 45mV |  |  |
| 230Vac / 100% load | 50mV Max | 30mV |  |  |

(VI). OVER VOLTAGE PROTECTION

Test result:

| Test condition | Spec. | Reading1 | Reading2 | Readimg3 |
| --- | --- | --- | --- | --- |
| 115Vac / 100% load | Vout*(110%~140%) | 115% |  |  |
| 230Vac / 100% load | Vout*(110%~140%) | 125% |  |  |

(VII). RISE TIME

Test result:

| Test condition | Spec. | Reading1 | Reading2 | Reading3 |
| --- | --- | --- | --- | --- |
| 115Vac / 100% load | 70mS Max | 30mS |  |  |
| 230Vac / 100% load | 70mS Max | 20mS |  |  |

(VIII). OVER CURRENT PROTECTION

Test result:

| Test condition | Spec. | Reading1 | Reading2 | Readimg3 |
| --- | --- | --- | --- | --- |
| 115Vac / 100% load | Iout*(101%~160%) | 120% |  |  |
| 230Vac / 100% load | Iout*(101%~140%) | 128% |  |  |

(IX). SHORT CIRCUIT PROTECTION

Test result:

| Test condition | Spec. | Reading1 | Reading2 | Readimg3 |
| --- | --- | --- | --- | --- |
| 115Vac / 100% load | Under 0.8W | OK |  |  |
| 230Vac / 100% load | Under 1.0W | OK |  |  |